DYF: Micro-hydro Assessment

- Define micro-hydro in one or two sentences.

- Can you measure head, flow and stride using buckets, timers, sticks, measuring tape and levels?  Y or N

- Use the equation and tables on the attached page to find the a) max power and b) net power for a system with 35 feet of available head for 200 feet of run and an average flow rate of 140 GPM.  Assume the efficiency to be 80%. Take 50% of the flow with 2” pipe and the following system components.

| Qty | Fitting |
| --- | --- |
| 2 | 90o |
| 5 | 45o |
| 3 | Gate Valve |
| 4 | T - Run |
| 0 | T - Branch |
| 20 | Adapter |

- On the back, diagram and label the basic physical components of a run of the river micro-hydro system.